TAITA TAVETA NATIONAL POLYTECHNIC

RESEARCH PROPOSAL

PROPOSED TITLE OF THE PROJECT	:ACOUSTIC LEVITATION SYSTEM

YOUR NAME				: AUSTIN MASUA

YOUR ADM NUMBER			: 20263

YOUR SUPERVISOR’S NAME		: MADAM CARO KISOMBE

ABSTRACT

This project proposal presents the design, fabrication, and experimental evaluation of a single-axis acoustic levitation system integrated with an optical sensor module for touchless water turbidity analysis. Traditional water quality sampling often introduces container-wall interference, sample contamination, and meniscus refraction errors during optical turbidity measurement. The proposed mechatronic system addresses these challenges by using 40 kHz ultrasonic transducers driven by an L298N module and an ATmega328P microcontroller to generate a stable acoustic standing wave capable of levitating small liquid droplets (10–50 µL) at ultrasonic pressure nodes. Turbidity and clarity are evaluated in real time using a focused red laser diode paired with a Light Dependent Resistor (LDR) optical detection circuit. By analyzing light attenuation across the levitated droplet, the system determines relative sample turbidity without physical vessel contact. This updated proposal document contains the complete Proteus ISIS schematic implementation guide, component connectivity netlist, and C++ microcontroller code for Timer1 40 kHz PWM generation and optical signal processing.

Table of Contents

1	INTRODUCTION	3

1.1	BACKGROUND AND CONTEXT	3

1.2	PROBLEM STATEMENT/RESEARCH PROBLEM	3

1.3	RESEARCH RATIONALE	3

1.4	RESEARCH AIMS, RESEARCH OBJECTIVES AND RESEARCH QUESTIONS	3

2	LITERATURE REVIEW	4

2.1	KEY CONCEPTS, THEORIES AND STUDIES	4

2.2	KEY DEBATES AND CONTROVERSIES	4

2.3	RESEARCH GAP	4

3	METHODOLOGY	5

3.1	RESEARCH DESIGN	5

3.2	DATA COLLECTION	5

3.3	SAMPLING	5

3.4	DATA ANALYSIS	5

3.5	PRACTICAL CONSIDERATIONS	5

4	PRACTICALITIES	6

4.1	IMPLICATIONS AND CONTRIBUTIONS TO KNOWLEDGE	6

4.1.1	PRACTICAL IMPLICATIONS	6

4.1.2	THEORETICAL IMPLICATIONS	6

4.2	PROJECT PLAN AND TIMELINES	6

4.3	BUDGET	6

5	REFERENCES	7

6	APPENDICES	8

1 INTRODUCTION

1.1 BACKGROUND AND CONTEXT

This project proposal presents the design, fabrication, and evaluation of a single-axis mechatronic acoustic levitation system for touchless chemical reactions, evaporation studies, and water turbidity analysis. Powered by an ATmega328P microcontroller and L298N driver operating 40 kHz ultrasonic transducers, the system generates stable standing waves to suspend 10–50 μL liquid droplets at pressure nodes, eliminating container-wall contamination and meniscus errors. An integrated optical detection module—combining a red laser diode and Light Dependent Resistor (LDR)—measures light attenuation across the levitated sample for contactless real-time turbidity and micro-droplet evaluation. Complete Proteus ISIS schematics, component netlists, and C++ PWM/signal processing code are provided.

1.2 PROBLEM STATEMENT/RESEARCH PROBLEM

Traditional liquid quality testing, biological sampling, and micro-scale chemical reactions rely on physical containment vessels, which introduce critical measurement errors and experimental artifacts. Physical container walls cause sample contamination, unwanted surface nucleation during crystallization, and meniscus-induced light refraction during optical analysis. Furthermore, conventional sampling equipment risks cross-contamination between successive chemical tests and suffers from material loss due to liquid adhesion on vessel walls.

Existing contact-based measurement techniques fail to provide a pristine environment for analyzing micro-volume liquid samples (10–50 μL) without interference from external solid boundaries. Consequently, there is a clear need for a low-cost, non-contact mechatronic platform capable of stably suspending liquid droplets in mid-air to enable containerless optical sensing, touchless micro-droplet mixing, and pure crystallization monitoring.

1.3 RESEARCH RATIONALE

This project demonstrates practical mechatronics implementation across mechanical fabrication, analog/digital electronics, power conversion, and embedded firmware. Developing a containerless water testing rig offers key educational and practical advantages:

- Elimination of Container Interference: Complete removal of cuvette wall interference, reflection, and sample contamination in micro-volume fluid evaluation.

- Cost-Effectiveness: High accessibility and low fabrication cost using readily available components (40 kHz transducers, L298N driver, ATmega328P/Arduino, LDR, and diode laser).

- Competency Demonstration: Demonstration of TVET Level 6 competencies in sensor integration, embedded C++ microcontroller programming, power electronics driving, and CAD mechanical design.

1.4 RESEARCH AIMS, RESEARCH OBJECTIVES AND RESEARCH QUESTIONS

Main Aim:

To design, fabricate, and validate a single-axis acoustic levitation water testing system using a 40 kHz ultrasonic driver and an LDR-laser optical sensor module.

Specific Research Objectives:

To design and construct a rigid mechanical frame and transducer housing with fine adjustment mechanisms for optimal acoustic resonance positioning.

To implement a 40 kHz PWM signal generator and L298N H-Bridge amplifier circuit driven by an ATmega328P microcontroller to establish stable acoustic standing waves.

To integrate a laser diode and LDR sensor module aligned directly across an acoustic node to measure light attenuation across levitated liquid droplets.

To calibrate LDR voltage responses against known water turbidity samples and evaluate system stability and testing repeatability.

Research Questions:

How accurately can a single-axis frame be positioned to maintain acoustic resonance for 10–50 μL droplets?

What PWM configuration and driving voltage on the L298N module produce the standing acoustic pressure field required to overcome gravitational acceleration?

What is the relationship between optical light attenuation across a levitated droplet and sample turbidity?

How stable and repeatable are turbidity measurements taken in mid-air compared to traditional contact-based methods?

2 LITERATURE REVIEW

2.1 KEY CONCEPTS, THEORIES AND STUDIES

Acoustic Levitation Fundamentals:

Acoustic levitation relies on acoustic radiation pressure produced by intense standing ultrasonic waves. When two identical 40 kHz transducers face each other separated by a distance D that is an integer multiple of half the acoustic wavelength (λ/2):

D=n×(2λ​)whereλ=fv​

At an ambient sound speed of v≈343 m/s and frequency f=40 kHz, the wavelength λ≈8.575 mm, yielding half-wavelength node spacing of approximately 4.29 mm. Droplets positioned near the velocity nodes experience a net restoring acoustic force balancing gravitational acceleration.

Optical Turbidity Sensing Principle:

Turbidity measures fluid clarity based on light scattering caused by suspended solids. According to the Beer-Lambert law, light transmission (I) through a medium decays exponentially with particle concentration (c) and path length (x):

I=I0​×e−α⋅c⋅x

In this project, a focused red laser diode (650 nm, 5mW) projects a narrow light beam directly through the acoustic node holding the levitated water droplet. The transmitted light strikes an LDR connected in a voltage divider circuit. As turbidity increases, light scattering reduces light intensity reaching the LDR, raising its resistance and altering the analog output voltage read by the ATmega328P ADC.

2.2 KEY DEBATES AND CONTROVERSIES

Standard water turbidity analysis relies on nephelometry inside glass or plastic cuvettes (ISO 7027-1 standard). A major debate in micro-fluidic analysis centers on boundary interactions:

Contact-Based Cuvettes: Provide fixed path lengths but introduce meniscus curvature refraction, surface wetting losses, and wall scattering.

Containerless Levitation: Eliminates container interference, but introduces challenges regarding droplet positional stability, acoustic streaming, and evaporative volume reduction during prolonged exposure.

2.3 RESEARCH GAP

While high-end acoustic levitators exist for pharmaceutical crystallization, they rely on costly transducers and complex array drivers. Conversely, low-cost open-source levitators (e.g., TinyLev) focus primarily on physical demonstration without quantitative analytical sensing. A clear gap exists for an accessible, low-cost TVET-level mechatronic platform that integrates an acoustic levitator directly with an optical attenuation sensor for quantitative real-time fluid analysis.

3 METHODOLOGY

3.1 RESEARCH DESIGN

This project adopts an experimental mechatronic research and development design divided into three primary phases: hardware co-design/simulation, embedded software implementation, and physical prototype validation.

3.2 DATA COLLECTION

Data collection involves measuring raw ADC voltage output from the LDR potential divider circuit across varying liquid clarity conditions.

Hardware Schematic & Circuit Netlist (Proteus ISIS):

U1 (ATmega328P / ARDUINO UNO): Microcontroller executing Timer1 Fast PWM mode (16 MHz external crystal oscillator with 22pF ceramic capacitors).

U2 (L298N Module / Driver IC): Dual Full-Bridge Driver boosting 5V PWM logic to 12V–16V peak-to-peak transducer excitation.

TR1, TR2 (TRANSDUCER / PIEZO): Represented in Proteus using ultrasonic transducer models or equivalent 40kHz piezo elements (1.5nF - 2.2nF capacitive load).

LDR1 (TORCH_LDR / LDR): Light Dependent Resistor model exposed to a variable light source in Proteus.

D1 (LASER DIODE / LED-RED): Red laser diode module powered via digital control pin D4 through a 220$\Omega$ current-limiting resistor.

R1 (10 kΩ Resistor): Fixed pull-down resistor forming the potential divider with LDR1 connected to analog pin A0.

LCD1 (LM016L 16x2 LCD): Alphanumeric LCD display connected in 4-bit parallel mode to digital pins D2, D3, D4, D5, D11, D12 for real-time turbidity display.

3.3 SAMPLING

Deionized water mixed with controlled concentrations of fine particulate matter simulates varying turbidity levels. Liquid droplets ranging from 10 μL to 50 μL are introduced into velocity nodes using a micropipette. The embedded algorithm records 50 consecutive ADC readings per evaluation cycle to filter out ambient noise and optical flicker.

3.4 DATA ANALYSIS

The ATmega328P ADC converts the LDR potential divider voltage into a 10-bit digital value (0–1023). Light clarity percentage is calculated via inverted mapping:

Clarity (%)=[1023.01023.0−rawADC​]×100.0

The result is constrained between 0.0% and 100.0%. Evaluated samples are categorized on the 16x2 LCD based on a safety consumption threshold (> 90.0% clarity required for safe drinking water status).

3.5 PRACTICAL CONSIDERATIONS

Single-axis vertical levitation is restricted to 10–50 μL liquid volumes. High sound intensity levels near 40 kHz require secure mechanical mounting to prevent structural resonance. Sensor calibration is performed in enclosed ambient light conditions to minimize external lighting interference.

4 PRACTICALITIES

4.1 IMPLICATIONS AND CONTRIBUTIONS TO KNOWLEDGE

4.1.1 PRACTICAL IMPLICATIONS

Provides a functional, portable, non-contact liquid analysis platform applicable to field water testing, micro-fluidic chemical evaluation, and educational physics demonstrations.

4.1.2 THEORETICAL IMPLICATIONS

Validates the practical application of acoustic radiation force equations and the Beer-Lambert law on containerless micro-droplet targets within a low-cost mechatronic system.

4.2 PROJECT PLAN AND TIMELINES

The project will be executed over a 12-week schedule:

| Phase / Activity | Duration | Start Week | Completion Week |
| --- | --- | --- | --- |
| Literature Review & System Design | 2 Weeks | Week 1 | Week 2 |
| Mechanical Frame Fabrication & Sourcing | 2 Weeks | Week 3 | Week 4 |
| Electronics Driver Assembly & 40kHz PWM Setup | 2 Weeks | Week 5 | Week 6 |
| Laser-LDR Optical Module Integration | 2 Weeks | Week 7 | Week 8 |
| Acoustic Levitation & Turbidity Testing | 2 Weeks | Week 9 | Week 10 |
| Data Analysis & Final Report Writing | 2 Weeks | Week 11 | Week 12 |

4.3 BUDGET

Estimated component and material costs in Kenyan Shillings (KSh):

| Item | Component Description | Specification / Model | Qty | Unit Cost (KSh) | Total Cost (KSh) |
| --- | --- | --- | --- | --- | --- |
| 1 | Ultrasonic Transducers | 40 kHz Piezo Sensors (Pair) | 2 | 600 | 1,200 |
| 2 | Motor Driver Module | L298N Dual H-Bridge Driver | 1 | 500 | 500 |
| 3 | Microcontroller Board | ATmega328P / Arduino Board | 1 | 1,200 | 1,200 |
| 4 | Optical Sensor Setup | 5mW 650nm Red Laser + LDR Module | 1 | 400 | 400 |
| 5 | Power Supply Unit | 12V 3A DC Adapter | 1 | 800 | 800 |
| 6 | Frame Fabrication | Acrylic Sheet / Threaded Rods / Hardware | 1 | 1,500 | 1,500 |
| 7 | Miscellaneous Electronics | Breadboard, Resistors, Wires, Connectors | 1 | 600 | 600 |
| TOTAL | ESTIMATED BUDGET |  |  |  | KSh 6,200 |

5 REFERENCES

Andrade, M. A., & Marzo, A. (2018). Acoustic levitation: Principles and applications. Physics Reports, 747, 1-48

ISO 7027-1:2016. Water quality — Determination of turbidity — Part 1: Quantitative methods. International Organization for Standardization.

Marzo, A., Barnes, A., & Drinkwater, B. W. (2017). TinyLev: A low-cost 3D-printed acoustic levitator for research and education. Review of Scientific Instruments, 88(8), 085105.

Microchip Technology. (2020). ATmega328P 8-bit AVR Microcontroller Datasheet (DS40002061B).

6 APPENDICES

Appendix A: Embedded C++ Firmware Source Code

C++

#include <LiquidCrystal.h>

// LCD Connections: RS=12, E=11, D4=5, D5=4, D6=3, D7=2LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const int LDR_PIN = A0;const float CONSUMPTION_THRESHOLD = 90.0; // Strictly > 90.0% is required for safe drinking water

void setup() {

// -------------------------------------------------------------

// 1. SETUP 40kHz PWM ON TIMER1 (PINS D9 & D10)

// -------------------------------------------------------------

pinMode(9, OUTPUT);    // OC1A - OUT1 driver signal

pinMode(10, OUTPUT);   // OC1B - OUT2 driver signal

pinMode(8, OUTPUT);    // ENA  - L298N Channel A Enable

digitalWrite(8, HIGH); // Drive ENA HIGH to enable the L298N

TCCR1A = 0;

TCCR1B = 0;

TCNT1  = 0;

TCCR1A = (1 << COM1A1) | (1 << COM1B1) | (1 << COM1B0);

TCCR1B = (1 << WGM13) | (1 << CS10);

ICR1   = 200;                                           // 40kHz signal

OCR1A  = 100;                                           // 50% duty cycle

OCR1B  = 100;

// -------------------------------------------------------------

// 2. SETUP LCD DISPLAY

// -------------------------------------------------------------

lcd.begin(16, 2);

lcd.print("Water Tester");

delay(1000);

lcd.clear();

}

void loop() {

int rawADC = analogRead(LDR_PIN);

// Inverted Mapping:

// As light INCREASES, LDR resistance drops, rawADC drops near 0.

// (1023.0 - rawADC) converts 0 ADC -> 100.0% Clarity.

float clarityPercent = ((1023.0 - rawADC) / 1023.0) * 100.0;

// Constrain range safely between 0.0% and 100.0%

if (clarityPercent > 100.0) clarityPercent = 100.0;

if (clarityPercent < 0.0)   clarityPercent = 0.0;

// Line 1: Display Percentage

lcd.setCursor(0, 0);

lcd.print("Clarity: ");

lcd.print(clarityPercent, 1);

lcd.print("%   ");

// Line 2: Evaluate > 90.0% threshold

lcd.setCursor(0, 1);

if (clarityPercent > CONSUMPTION_THRESHOLD) {

lcd.print("SAFE FOR DRINK  ");

} else {

lcd.print("UNSAFE / TREAT  ");

}

delay(50); // Prevent LCD screen flicker

}